Supplementary file

Electrocoagulation Technology for Oil Removal in Food Wastewater: Research Trends and Patent Landscape Analysis

J.A.L. Sooa, T.T. Henga, L. S. Tana*, K.H. Ahmada, N. F. Shoparweb, M.M. Nasefa, S. Suakantoc

a Department of Chemical and Environmental Engineering, Malaysia - Japan International Institute of Technology, Universiti Teknologi Malaysia, 54100 Kuala Lumpur, Malaysia.

b Faculty of Bioengineering and Technology, Jeli Campus, Universiti Malaysia Kelantan, 17600 Jeli Kelantan, Malaysia.

c Department of Information System, Telkom University, Bandung 40257, Indonesia.

Table S1 presents a comprehensive compilation of electrocoagulation studies applied to food industry wastewater treatment, including wastewater source, characteristics, key operating parameters, and pollutant removal performance reported in the reviewed literature.

Table S1	 Application of electrocoagulation technology in food wastewater treatment

| Food samples source | Wastewater characteristics | Parameters | Removal percentage |  |  | Reference |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Oil | COD |  |  |
| Restaurant | O&G: 325 mg/L COD: 1370 mg/L pH: 6.75 | Electrode material: Aluminium Time: 15 min Electric applied: 2.49 F/m3 (charge loading) | 98.3% |  | 60.7% | [1] |
| Chinese restaurant | O&G: 120 – 712 mg/L COD: 606 – 2400 mg/L Conductivity: 293 – 422 μS/cm pH: 6.40 – 6.67 | Electrode material: Aluminium Time: 9.0 L/h (flow rate) Electric applied: 4.00 – 4.97 F/m3 (Charge loading) | 0.28 – 3.8 mg/L | 139 – 525 mg/L |  | [2] |
| Western restaurant | O&G: 309 – 332 mg/L COD: 1940 – 2190 mg/L Conductivity: 309 – 441 μS/cm pH: 6.94 – 7.35 | Electrode material: Aluminium Time: 9.0 L/h (flow rate) Electric applied: 4.97 F/m3 (Charge loading) | 1.17 – 6.6 mg/L | 320 – 685 mg/L |  | [2] |
| American fast-food | O&G: 355 – 402 mg/L COD: 1150 – 4240 mg/L Conductivity: 576 – 706 μS/cm pH: 6.28 – 7.06 | Electrode material: Aluminium Time: 9.0 L/h (flow rate) Electric applied: 4.97 – 9.95 F/m3 (Charge loading) | 1.04 – 5.7 mg/L | 367 – 2420 mg/L |  | [2] |

*Chemical oxygen demand (COD), Fats, oils and grease (FOG)

Table S1	 Application of electrocoagulation technology in food wastewater treatment (continued)

| Source food sample | Characteristic of wastewater | Parameters | Removal percentage |  | Reference |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Oil | COD |  |
| Student canteen | O&G: 1090 – 1500 mg/L COD: 2450 – 2760 mg/L Conductivity: 476 – 701 μS/cm pH: 6.80 – 7.07 | Electrode material: Aluminium Time: 9.0 L/h (flow rate) Electric applied: 1.67 – 4.97 F/m3 (Charge loading) | 0.8 – 8.4 mg/L | 153 – 453 mg/L | [2] |
| UC bistro | O&G: 140 – 410 mg/L COD: 1500 – 1740 mg/L Conductivity: 341 – 514 μS/cm pH: 7.63 – 8.22 | Electrode material: Aluminium Time: 9.0 L/h (flow rate) Electric applied: 6.63 – 7.46 F/m3 (Charge loading) | 7.5 – 7.9 mg/L | 357 – 443 mg/L | [2] |
| Egg processing industry | COD: 4150 ± 81.8 mg/L | Electrode material: Stainless steel Time: 35 ± 1.3 min Electric applied: 15 V | 92 ± 1.65% |  | [3] |
| Canteen | O&G: 100 – 250 mg/L COD: 500 – 150 mg/L | Electrode material: Iron Time: 30 min pH: 6 Supporting electrolyte: 720 μS/cm Electric applied: 12.0 A/m2 | 95% | 75% | [4] |

*Chemical oxygen demand (COD), Fats, oils and grease (FOG)

Table S1	 Application of electrocoagulation technology in food wastewater treatment (continued)

| Source food sample | Characteristic of wastewater |  | Parameters | Removal percentage |  | Reference |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Oil | COD |  |
| Potato chips industry | COD: 2200 – 2800 mg/L Conductivity: 1.90 – 2.40 mS/cm pH: 6.2 – 6.5 |  | Electrode material: Aluminium Time: < 40 min, pH: 4 Electric applied: 200 A/m2 | 60% |  | [5] |
| Slaughterhouse | O&G: 1500 – 1800 mg/L COD: 26000 – 29000 mg/L Conductivity: 199 mS/cm pH: 6.7 |  | Electrode material: Aluminium Time: 7.5 min (oil) 25 min (COD) pH: 2 Electric applied: 150 A/m2 | 90% | 93% | [6] |
| Dairy industry | O&G: 4570 mg/L COD: 18300 mg/L Conductivity: 1200 μS/cm pH: 6.0 – 7.5 | Electrode material: Iron Time: 1 min pH: 7 Supporting electrolyte: 0.3 g/L Electric applied: 6 A/m2 |  | 99% | 98% | [7] |
| Cookies and pasta industry | COD: 7500 mg/L | Electrode material: Aluminium pH: 4 Electric applied: 182 A/m2 |  | 80% |  | [8] |

*Chemical oxygen demand (COD), fats, oils and grease (FOG)

Table S1	 Application of electrocoagulation technology in food wastewater treatment (continued)

| Source food sample |  | Characteristic of wastewater |  | Parameters |  |  |  | Removal percentage |  |  |  | Reference |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Oil | COD |  |  |  |  |
| Slaughter house |  | O&G: 853 ± 119 mg/L COD: 3340 ± 180 mg/L Conductivity: 473 ± 14 μS/cm pH: 6.15 – 6.45 |  | Electrode material: Mild steel (Iron) Time: 60 min Electric applied: 0.3 A |  |  |  | 99 ± 1% | 82 ± 2% |  |  | [9] |  |
|  |  |  |  | Electrode material: Aluminium Time: 15 min (oil) 25 min (COD) pH: 2 Electric applied: 150 A/m2 |  |  |  | 95% | 85% |  |  | [9] |  |
| Slaughter house | O&G: 853 ± 119 mg/L COD: 1270 ± 30 mg/L |  |  |  | Electrode material: Iron Time: 60 min Electric applied: 190 A/m2 | 85.1% |  |  |  |  | [19] |  |  |
| Food beverage |  | COD: 3460 mg/L Conductivity: 137 μS/cm pH: 5.81 | Electrode material: Iron Time: 90 min Electric applied: 50 A/m2 |  |  |  | 40.2% |  |  |  |  |  | [10] |
|  |  | COD: 3160 mg/L Conductivity: 131 μS/cm pH: 5.79 | Electrode material: Aluminium Time: 90 min Electric applied: 50 A/m2 |  |  |  | 19.9% |  |  |  |  |  | [10] |

*Chemical oxygen demand (COD), fats, oils and grease (FOG)

Table S1	 Application of electrocoagulation technology in food wastewater treatment (continued)

| Source food sample | Characteristic of wastewater | Parameters | Removal percentage |  | Reference |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Oil | COD |  |
| Cereal products | COD: 369 mg/L Conductivity: 108 μS/cm pH: 5.71 | Electrode material: Iron Time: 90 min Electric applied: 50 A/m2 |  | 23.3% | [10] |
|  | COD: 369 mg/L pH: 5.53 | Electrode material: Aluminium Time: 90 min Electric applied: 50 A/m2 |  | 22.8% | [10] |
| Meat processing | COD: 3010 mg/L Conductivity: 12400 μS/cm pH: 8.56 | Electrode material: Iron Time: 90 min Electric applied: 50 A/m2 |  | 20.3% | [10] |
|  | COD: 3310 mg/L Conductivity: 12800 μS/cm pH: 8.71 | Electrode material: Aluminium Time: 90 min Electric applied: 50 A/m2 |  | 33.5% | [10] |
| Slaughterhouse | COD: 1410 mg/L Conductivity: 334 μS/cm pH: 6.32 | Electrode material: Iron Time: 90 min Electric applied: 50 A/m2 |  | 85.1% | [10] |

*Chemical oxygen demand (COD)

Table S1	 Application of electrocoagulation technology in food wastewater treatment (continued)

| Source food sample | Characteristic of wastewater | Parameters | Removal percentage |  | Reference |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Oil | COD |  |
| Baker’s yeast industry | COD: 2485 mg/L Conductivity: 19.20 mS/cm pH: 7.8 | Electrode material: Iron/Aluminium Time: 50 min pH: 7 (Iron)/ 6.5 (Aluminium) Electric applied: 70 A/m2 |  | 69% (Iron) 71% (Al) | [11] |
| Almond industry | COD: 6300 ppm Conductivity: 2.3 mS/cm pH: 5.7 | Electrode material: Aluminium-Iron Electric applied: 41 A/m2 |  | 81% | [12] |
| Winery industry | COD: 25200 – 28640 mg/L Conductivity: 3.5 mS/cm pH: 5.2 | Electrode material: Aluminium Time: 120 min pH: 5.2 Electric applied: 300 A/m2 |  | 48.5% | [13] |
|  |  | Electrode material: Iron Time: 90 min pH: 7 Electric applied: 300 A/m2 |  | 46.6% | [13] |

*Chemical oxygen demand (COD)

Table S1	 Application of electrocoagulation technology in food wastewater treatment (continued)

| Source food sample | Characteristic of wastewater | Parameters | Removal percentage |  | Reference |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Oil | COD |  |
| Dairy industry | COD: 8960 ± 16.4 mg/L Conductivity: 1082.2 ± 8.5 μS pH: 7.10 ± 0.12 | Electrode material: Aluminium Time: 60 min Electric applied: 3 A |  | < 39% | [14] |
| Sweet-snacks industry | COD: 9720 ± 17.2 mg/L Conductivity: 1073.7 ± 12.1 μS pH: 5.64 ± 0.12 | Electrode material: Aluminium Time: 60 min Electric applied: 3 A |  | < 59% | [14] |
| Ice-cream industry | COD: 11900 ± 21.3 mg/L Conductivity: 794.4 ± 9.1 μS pH: 6.25 ± 0.15 | Electrode material: Aluminium Time: 60 min Electric applied: 3 A |  | < 50% | [14] |
| Egg processing industry | COD: 3200 – 4300 mg/L Conductivity: 0.455 – 0.614 mS/cm pH: 7.34 – 7.95 | Electrode material: Aluminium Time: 30 min pH: 6 Supporting electrolyte: 1.5 g/L NaCl Electric applied: 200 A/m2 |  | 89% | [15] |

*Chemical oxygen demand (COD)

Table S1	 Application of electrocoagulation technology in food wastewater treatment (continued)

| Source food sample | Characteristic of wastewater | Parameters | Removal percentage |  | Reference |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Oil | COD |  |
| Cheese industry | COD: 15500 mg/L Conductivity: 17.00 mS/cm pH: 5.2 | Electrode material: Iron Time: 20 min pH: 4.54 Electric applied: 600 A/m2 |  | 86.4% | [16] |
| Restaurant | O&G: 180 – 280 mg/L Conductivity: 1625 – 3480 μS/cm pH: 5 – 6 | Electrode material: Aluminium (anode), Stainless steel (cathode) Time: 34 min Supporting electrolyte: 2980 μS/cm (2.2%) salinity Electric applied: 43 A/m2 | 95.8 ± 0.6% |  | [17] |
| Baker’s yeast industry (Molasses) | COD: 4150 mg/L pH: 8.5 | Electrode material: Iron (anode) Copper (cathode) Time: 3.5 hour Electric applied: 330 A/m2 |  | 45% | [18] |
| Olive oil processing industry | COD: 1344 ± 22 mg/L Conductivity: 1260 ± 32 μS/cm pH: 6.9 | Electrode material: Aluminium Time: 120 min Electric applied: 12.9 A/m2 | 62.5% |  | [19] |

*Chemical oxygen demand (COD), fats, oils and grease (FOG)

Table S1	 Application of electrocoagulation technology in food wastewater treatment (continued)

| Source food sample | Characteristic of wastewater | Parameters | Removal percentage |  | Reference |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Oil | COD |  |
| Edible oil | COD: 4000 mg/L | Electrode material: Stainless steel Time: 80 min pH: 7 Supporting electrolyte: 150 mg/L Electric applied: 41.1 A/m2 | 97.71% (Fatty acid) | 94.6 ± 0.2% | [20] |
| Slaughterhouse |  | FOG: ~100 – 190 mg/L COD: ~800 – 1050 mg/L | Electrode material: Aluminium Time: > 5 min Electric applied: ~ 30 A/m2 | 94% | 59% | [21] |
| Food industry | COD: 17000 ± 550 mg/L Conductivity: 3.48 ± 0.02 mS/cm pH: 4.43 ± 0.08 | Electrode material: Iron Time: 21.36 min pH: 10.0 Electric applied: 860 A/m2 | 29.41% |  | [22] |
| Potato processing industry | COD: 500 mg/L Conductivity: 3.40 mS/cm pH: 7.8 | Electrode material: Aluminium Time: 20 min pH: 7.8 Electric applied: 15 A/m2 | 70% |  | [23] |

*Chemical oxygen demand (COD), fats, oils and grease (FOG)

Table S1	 Application of electrocoagulation technology in food wastewater treatment (continued)

| Source food sample |  | Characteristic of wastewater |  | Parameters |  | Removal percentage |  |  |  |  | Reference |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Oil |  | COD |  |  |  |  |  |
| Vinegar industry |  | COD: 10000 mg/L Conductivity: 2.00 mS/cm pH: 4.00 |  | Electrode material: Iron Time: 75 min pH: 9 Electric applied: 225 A/m2 |  |  |  | 93.58% |  |  | [24] |  |  |
|  |  |  |  | Electrode material: Aluminium Time: 30 min pH: 4 Electric applied: 200 A/m2 |  |  |  | 90.91% |  |  | [24] |  |  |
| Olive oil extraction industry | COD: 20124 ± 72.6 mg/L Conductivity: 162.80 ± 2.31 mS/cm pH: 4.40 ± 0.04 |  |  |  | Electrode material: Aluminium Time: 60 min pH: 4.40 ± 0.05 Supporting electrolyte: Electric applied: 150 A/m2 |  |  |  | 71.90 ± 2.71% |  |  | [25] |  |
| Restaurant |  |  | - |  | Electrode material: Carbon Time: 90 min Electric applied: 1 A |  |  |  |  | 40.7% |  |  | [26] |

*Chemical oxygen demand (COD)

Table S1	 Application of electrocoagulation technology in food wastewater treatment (continued)

| Source food sample | Characteristic of wastewater | Parameters | Removal percentage |  | Reference |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Oil | COD |  |
| Abo-Saleh Restaurant | O&G: 264 mg/L COD: 15120 mg/L Conductivity: 765 μS/cm pH: 6.93 | Electrode material: Aluminium Time: 60 min Electric applied: 400 A/m2 | 92.42% | 89.15% | [27] |
| El-Ezz El-Soury restaurant | O&G: 1184 mg/L COD: 5440 mg/L Conductivity: 1139 μS/cm pH: 7.26 | Electrode material: Aluminium Time: 60 min Electric applied: 400 A/m2 | 100% | 81.6% | [27] |
| Hadramawt restaurant | O&G: 234 mg/L COD: 5280 mg/L Conductivity: 5330 μS/cm pH: 6.7 | Electrode material: Aluminium Time: 60 min Electric applied: 400 A/m2 | 100% | 74.5% | [27] |
| KFC restaurant | O&G: 218 mg/L COD: 3808 mg/L Conductivity: 1023 μS/cm pH: 7 | Electrode material: Aluminium Time: 60 min Electric applied: 400 A/m2 | 94.5% | 68.5% | [27] |

*Chemical oxygen demand (COD), Fats, oils and grease (FOG)

Table S1	 Application of electrocoagulation technology in food wastewater treatment (continued)

| Source food sample | Characteristic of wastewater | Parameters | Removal percentage |  | Reference |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Oil | COD |  |
| Pizza Party restaurant | O&G: 286 mg/L COD: 5440 mg/L Conductivity: 400 μS/cm pH: 8.02 | Electrode material: Aluminium Time: 60 min Electric applied: 400 A/m2 | 87.76% | 92.79% | [27] |
| Olive oil extraction industry | COD: 2.5 g/L pH: 6.5 | Electrode material: Iron Time: 45 min Electric applied: 50 A/m2 |  | 80% | [28] |
| Fruit juice industry | COD: 14800 mg/L Conductivity: 436 μS/cm pH: 7.43 | Electrode material: Aluminium (anode) Stainless steel (cathode) Time: 40 min pH: 7.43 Electric applied: 400 A/m2 |  | 83.78% | [29] |
| Chocolate industry | COD: 1732 mg/L Conductivity: 1680 μS/cm pH: 6.4 | Electrode material: Aluminium Time: 60 min Electric applied: 3.16 A |  | 63% | [30] |

*Chemical oxygen demand (COD), fats, oils and grease (FOG)

Table S1	Application of electrocoagulation technology in food wastewater treatment (continued)

| Source food sample | Characteristic of wastewater | Parameters | Removal percentage |  | Reference |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Oil | COD |  |
| Hainanese chicken rice street food vendor | FOG: 310 mg/L ± 12% COD: 40.6 g/L ± 15% Conductivity: 0.68 mS/cm ± 4% + 5 g/L NaCl pH: 5.6 ± 7% | Electrode material: Aluminium Time: 4 min Electric applied: 200 A/m2 | 84% | 95% | [31] |
| Noodles and dumplings street food vendor | FOG: 178 mg/L ± 8% COD: 27.3 g/L ± 10% Conductivity: 0.95 mS/cm ± 5% pH: 7.2 ± 10% |  | 86% | 98% | [31] |
| Slaughterhouse | FOG: 280 – 1668 mg/L COD: 3968 – 5239 mg/L Conductivity: 1.36 – 3.04 mS/cm pH: 6.19 – 7.24 | Electrode material: Iron Time: 74 min pH: 3.05 Electric applied: 66.9 A/m2 | 99% | 92.4% | [32] |

*Chemical oxygen demand (COD), fats, oils and grease (FOG)

Table S1 Application of electrocoagulation technology in food wastewater treatment (continued)

| Source food sample | Characteristic of wastewater | Parameters | Removal percentage |  | Reference |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Oil | COD |  |
| Slaughterhouse | O&G: 878.80 mg/L COD: 4381 mg/L Conductivity: 497.40 μS/cm pH: 6.26 | Electrode material: Aluminium Time: 20 min Electric applied: 200 A/m2 | 97% | 88% | [33] |
|  |  | Electrode material: Iron Electric applied: 200 A/m2 | 99% | 90% | [33] |
| Confectionary Wastewater | COD: 2020 mg/L Conductivity: 4500 μS/cm pH: 8.0 | Electrode material: Iron Time: 75 min Electric applied: 90 A/m2 |  | 77% | [34] |
| Dairy industry | COD: 75300 – 80300 mg/L Fat: 15 – 18% w/w Conductivity: 2.5 – 4.8 mS/cm pH: 6.2 – 6.9 | Electrode material: Aluminium Time: 120 min pH: 6.6 Electric applied: 400 A/cm2 |  | 40.5% | [35] |
| Tomato Paste | COD: 1260 mg/L Conductivity: 1490 μS/cm pH: 6.84 | Electrode material: Aluminium, Iron, Graphite, Stainless Steel (SS) Time: 120 min Electric applied: 75 A/m2 |  | 81.0% (Al) 51.1% (Fe) 55.6% (Graphite) 50.5% (SS) | [36] |

*Chemical oxygen demand (COD), fats, oils and grease (FOG)

Table S1 Application of electrocoagulation technology in food wastewater treatment (continued)

| Source food sample | Characteristic of wastewater |  | Parameters | Removal percentage |  | Reference |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Oil | COD |  |
| Palm Oil Mill Effluents | COD: n/a pH: n/a |  | Electrode material: Aluminium Time: 60 min Electric applied: 132.74 A/m2 |  | 96.8% | [37] |
| Citric Acid Production Wastewater | COD: 21000 mg/L Conductivity: 436 μS/cm pH: 2.00 |  | Electrode material: Iron Time: 120 min Electric applied: 20.0 V |  | 98% | [38] |
| Egg Wash Wastewater |  | COD: 1100 mg/L pH: 6.00 | Electrode material: Iron Time: 60 min Electric applied: 200 A/m2 |  |  | >90% | [39] |
| Palm Oil Mill Effluent |  | COD: 5440.80mg/L pH: 8.32 | Electrode material: Graphite (Anode) and Stainless Steel (Cathode) Time: 9 min Electric applied: 60 A/cm2 |  |  | 97.22% | [40] |

*Chemical oxygen demand (COD), **Phenolic compounds percentage removal

Table S1	Application of electrocoagulation technology in food wastewater treatment (continued)

| Source food sample | Characteristic of wastewater | Parameters | Removal percentage |  | Reference |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Oil | COD |  |
| Pineapple Fruit Waste | COD: 1400 mg/L pH: 13.4 | Electrode material: Aluminium Time: 90 min Electric applied: 350 A/m2 |  | 84.8% | [41] |

*Chemical oxygen demand (COD)

References:

- G. Chen, X. Chen, P.L. Yue, Electrocoagulation and electroflotation of restaurant wastewater , Journal of Environmental Engineering, 126 (2000) 858-863. https://doi.org/10.1061/(ASCE)0733-9372(2000)126:9(858)

- X. Chen, G. Chen, P. L. Yue, Separation of pollutants from restaurant wastewater by electrocoagulation, Separation and Purification Technology, 19 (2000) 65-76. https://doi.org/10.1016/S1383-5866(99)00072-6.

- L. J. Xu, B. W. Sheldon, D. K. Larick, R. E. Carawan, Recovery and utilization of useful by-products from egg processing wastewater by electrocoagulation, Poultry science, 81 (2002) 785-792. https://doi.org/10.1093/ps/81.6.785

- X. Xu & X. Zhu, Treatment of refectory oily wastewater by electro-coagulation process, Chemosphere, 56 (2004) 889-894. https://doi.org/10.1016/j.chemosphere.2004.05.003

- M. Kobya, H, Hiz, E, Senturk, C. Aydiner, E. Demirbas, Treatment of potato chips manufacturing wastewater by electrocoagulation, Desalination, 190 (2006)
201-211. https://doi.org/10.1016/j.desal.2005.10.006

- M. Kobya, E. Senturk, M. Bayramoglu, Treatment of poultry slaughterhouse wastewaters by electrocoagulation, Journal of Hazardous Materials, 133 (2006) 172-176. https://doi.org/10.1016/j.jhazmat.2005.10.007

- I.A. Şengil & M. Ozacar, Treatment of dairy wastewaters by electrocoagulation using mild steel electrodes, Journal of Hazardous Materials, 137 (2006) 1197-1205. https://doi.org/10.1016/j.jhazmat.2006.04.009

- G. Roa-Morales, E. Campos-Medina, J. Aguilera-Cotero, B. Bilyeu, C. Barrera-Díaz, Aluminum electrocoagulation with peroxide applied to wastewater from pasta and cookie processing, Separation and Purification Technology, 54 (2007) 124-129. https://doi.org/10.1016/j.seppur.2006.08.025.

- M. Asselin, M., P. Drogui, H. Benmoussa, J. F. Blais, Effectiveness of electrocoagulation process in removing organic compounds from slaughterhouse wastewater using monopolar and bipolar electrolytic cells, Chemosphere 72 (2008) 1727-1733. https://doi.org/10.1016/j.chemosphere.2008.04.067.

- P. Drogui, M. Asselin, S. K. Brar, H. Benmoussa, J. F. Blais, Electrochemical removal of pollutants from agro-industry wastewaters, Separation and Purification Technology 61 (2008) 301-310. https://doi.org/10.1016/j.seppur.2007.10.013

- M. Kobya & S. Delipinar, Treatment of the baker's yeast wastewater by electrocoagulation, Journal of Hazardous Materials, 154 (2008) 1133-1140. https://doi.org/10.1016/j.jhazmat.2007.11.019.

- D. Valero, J. M. Ortiz, V. García, E. Expósito, V. Montiel, A. Aldaz, Electrocoagulation of wastewater from almond industry, Chemosphere, 84 (2011) 1290-1295. https://doi.org/10.1016/j.chemosphere.2011.05.032.

- S. Kara, E. Gürbulak, M. Eyvaz, E. Yüksel, Treatment of winery wastewater by electrocoagulation process'=, Desalination and Water Treatment, 51 (2013) 5421-5429. https://doi.org/10.1080/19443994.2013.770223

- W. Qasim, A. Mane, Characterization and treatment of selected food industrial effluents by coagulation and adsorption techniques, Water resources and industry, 4 (2013) 1-12. https://doi.org/10.1016/j.wri.2013.09.005

- R. Sridhar, V. Sivakumar, J. P. Maran, K. Thirugnanasambandham, K, Influence of operating parameters on treatment of egg processing effluent by electrocoagulation process, International journal of environmental science and technology, 11 (2014) 1619-1630. https://doi.org/10.1007/s13762-013-0301-5.

- U. T. Un, A. Kandemir, N. Erginel, S.E. Ocal, Continuous electrocoagulation of cheese whey wastewater: an application of response surface methodology, Journal of Environmental Management, 146 (2014) 245-250. https://doi.org/10.1016/j.jenvman.2014.08.006

- M. Ji, X. Jiang, F. Wang, Mechanistic approach and response surface optimization of the removal of oil and grease from restaurant wastewater by electrocoagulation and electroflotation, Desalination and Water Treatment, 55 (2015) 2044-2052. https://doi.org/10.1080/19443994.2014.929034

- C. Tsioptsias, D. Petridis, N. Athanasakis, I. Lemonidis, S. Deligiannis, P. Samaras, Post-treatment of molasses wastewater by electrocoagulation and process optimization through response surface analysis, Journal of Environmental Management, 164 (2015) 104-113. https://doi.org/10.1016/j.jenvman.2015.09.007

- H. Ates, N. Dizge, H. C. Yatmaz, Combined process of electrocoagulation and photocatalytic degradation for the treatment of olive washing wastewater, Water Science and Technology, 75 (2017) 141-154. https://doi.org/10.2166/wst.2016.498

- A. Mirshafiee, A. Rezaee, R. S. Mamoory, A clean production process for edible oil removal from wastewater using an electroflotation with horizontal arrangement of mesh electrodes, Journal of cleaner production, 198 (2018) 71-79. https://doi.org/10.1016/j.jclepro.2018.06.201

- K. Sardari, J. Askegaard, Y. H. Chiao, S. Darvishmanesh, M. Kamaz, S. R. Wickramasinghe, Electrocoagulation followed by ultrafiltration for treating poultry processing wastewater, Journal of Environmental Chemical Engineering, 6 (2018) 4937-4944. https://doi.org/10.1016/j.jece.2018.07.022.

- G. Varank, S. Y. Guvenc, A. Demir, A comparative study of electrocoagulation and electro-Fenton for food industry wastewater treatment: Multiple response optimization and cost analysis, Separation Science and Technology, 53 (2018) 2727-2740. https://doi.org/10.1080/01496395.2018.1470643.

- S. Velİ, I. Özbay, B. Özbay, A. Arslan, E. Çebİ, Optimization of process variables for treatment of food industry effluents by electrocoagulation, Global Nest Journal, 20 (2018) 551-557. https://doi.org/10.30955/gnj.002640

- S. Yılmaz, E. E. Gerek, Y. Yavuz, A. S. Koparal, Treatment of vinegar industry wastewater by electrocoagulation with monopolar aluminum and iron electrodes and toxicity evaluation, Water Science and Technology, 78 (2018) 2542-2552. https://doi.org/10.2166/wst.2019.013.

- R. Niazmand, M. Jahani, S. Kalantarian, Treatment of olive processing wastewater by electrocoagulation: An effectiveness and economic assessment, Journal of Environmental Management, 248 (2019) 1-8. https://doi.org/10.1016/j.jenvman.2019.109262

- A. T. Adegoke, E. T. Abayomi, A preliminary study on the treatment of restaurant wastewater using electrocoagulation technique, Journal of Degraded and Mining Lands Management, 7 (2020) 2029-2033. http://dx.doi.org/10.15243/jdmlm.2020.072.2029

- K. H. El-Ezaby, M. I. El-Gammal, A. Y. Shaaban, Electrocoagulation treatment for wastewaters from some restaurants in New Damietta City-Egypt, Journal of Environmental Sciences, 49 (2020) 87-98. https://doi.org/10.21608/joese.2020.177139

- M. R. Khani, H. Mahdizadeh, K. Kannan, L. R. Kalankesh, B. Kamarehei, M. M.  Baneshi, Y. D. Shahamat, Olive Mill Wastewater (OWM) Treatment by Hybrid Process of Electrocoagulation/Catalytic Ozonation and Biodegradation, Environmental Engineering & Management Journal, 19 (2020), 1401-1410. http://dx.doi.org/10.30638/eemj.2020.130.

- K. H. El-Ezaby, M. I. El-Gammal, A. Y. Shaaban, Using electro-and alum coagulation technologies for treatment of wastewater from fruit juice industry in New Damietta City, Egypt, Environmental Monitoring and Assessment, 193 (2021), 1-16. https://doi.org/10.1007/s10661-021-09149-0.

- V. García-Orozco, G. Roa-Morales, I. Linares-Hernández, I. Serrano-Jimenes, M. Salgado-Catarino, R. Natividad, Electrocoagulation of a chocolate industry wastewater in a Downflow column electrochemical reactor, Journal of Water Process Engineering, 42 (2021) 1-8. https://doi.org/10.1016/j.jwpe.2021.102057.

- W. Khanitchaidecha, Y. K. Ratananikom, B. Yangklang, S. Intanoo, K. Sing-Aed, A. Nakaruk, Application of Electrocoagulation in Street Food Wastewater, Water, 14 (2022), 1-8. https://doi.org/10.3390/w14040655

- P. V. Ngobeni, L. Gutu, M. Basitere, T. Harding, D. Ikumi, Poultry slaughterhouse wastewater treatment using an integrated biological and electrocoagulation treatment system: process optimisation using response surface methodology, Sustainability, 14 (2022) 1-13. https://doi.org/10.3390/su14159561.

- M. C. Potrich, E. D. S. A. Duarte, M. D. S. Sikora, R. D. Costa da Rocha, Electrocoagulation for nutrients removal in the slaughterhouse wastewater: comparison between iron and aluminum electrodes treatment, Environmental Technology, 43 (2022) 751-765. https://doi.org/10.1080/09593330.2020.1804464.

- H. Dostiyiev, Z. B. Gönder, A comparative investigation of chemical coagulation and electrocoagulation for the post-treatment of confectionery wastewater, International Journal of Environmental Science and Technology, 21 (2023) 3661-3674. https://doi.org/10.1007/s13762-023-05228-6.

- S. Elia, M. Stylianou, A. Agapiou, Combined EC/EO processes for treating goat cheese whey wastewater, Sustainable Chemistry and Pharmacy, 32 (2023) 1-15. https://doi.org/10.1016/j.scp.2022.100963.

- S. Aliye, A. Ceyhum, B. Zeynep, A. Hudaverdi, D. Nadir, Treatment of tomato paste wastewater by electrochemical and membrane processes: process optimization and cost calculation, Water Science and Technology, 89 (2024) 1879-1890. https://doi.org/10.2166/wst.2024.079.

- P. T. P. Aryanti, F. A. Nugroho, N. Anwar, F. Rusgiyarto, C. Phalakornkule, A. Kadier, Integrated bipolar electrocoagulation and PVC-based ultrafiltration membrane process for palm oil mill effluent (POME) treatment, Chemosphere, 347 (2024) 140637. https://doi.org/10.1016/j.chemosphere.2023.140637.

- P. Belibegli, H. E. G. Akbay, S. Arslan, B. Mazmanci, N. Dizge, N. Senthilkumar, D. Balakrishnan, Enhanced biogas yield in anaerobic digestion of citric acid wastewater by pre-treatment: The effect of calcium hydroxide precipitation and electrocoagulation process, Process Safety and Enviromental Protection, 184 (2024) 1344-1356. https://doi.org/10.1016/j.psep.2024.02.050.

- P. Bhatt, B. A. Engel, K. B. Shivaram, R. F. Turco, Z. Zhou, H. Simsek, Treatment and optimization of high-strength egg-wash wastewater effluent using electrocoagulation and electrooxidation methods, Chemosphere, 347 (2024), 140632. https://doi.org/10.1016/j.chemosphere.2023.140632.

- P. Khongkliang, K. Chalearmkul, K. Boonloh, N. Kanjanasombun, T. Darnsawat, J. Boonnorat, A. Kadier, P. T. P. Aryanti, C. Phalakornkule, Performance of combined organic precipitation, electrocoagulation, and electrooxidation in treating anaerobically treated palm oil mill effluents, Applied Water Science, 14 (2024) 222. 
https://doi.org/10.1007/s13201-024-02288-y.

- J. Kumar, I. Alam, A. Kumar, A. Kumar, S. K. Singh, S. P. Singh, C. Sharma, Sustainable utilization of pineapple fruit waste as a potential source of regenerated cellulose for the development of high-strength paper, Biomass and Bioenergy ,182 (2024) 107068. https://doi.org/10.1016/j.biombioe.2024.107068.